MRI-Guided Microwave Thermal Ablation

Formulation as a PDE-Constrained Optimal Control Problem

Free Final Time, State-Constrained Variational Formulation

Jing-Yuan Huang

270C Final Project Proposal

# MRI Surgical Robot Hardware

# 1. Problem Overview and Physical Setting

MRI-guided thermal ablation is the clinical procedure of using a controlled energy source (microwave, radiofrequency, laser, or focused ultrasound) to heat and destroy a tumor target while sparing surrounding healthy tissue. Real-time MRI temperature imaging (MR thermometry, via the proton resonance frequency shift) provides online feedback of the three-dimensional temperature field, making this a natural setting for closed-loop, feedback-optimal control.

General surgical procedures are shown as follows:

- Identify tumor area using 3D MRI volume scan

- Locate insertion point and depth from surgeon’s experience

- Use robotics system to align ablation probe to the target point

- Start ablation with MRI thermometry feedback

- Check ablation zone with volume scan

- Retract probe and finish

The goal is to create an ablation zone just big enough to cover the tumor area with roughly 1-cm boarder and prevent killing surrounding healthy tissue.

## 1.1 Spatial Coordinates

We denote the tissue domain by Ω ⊂ ℝ³ (or the cylindrical domain Ω ⊂ ℝ² in the axisymmetric (r, z) case). The boundary is ∂Ω, with inward unit normal n̂. The time horizon is t ∈ [0, t_f], where the final time tf>0 is a free optimization variable.

Cartesian: r = (x, y, z) ∈ Ω ⊂ ℝ³

Cylindrical (axisymmetric): r = (r, θ, z)

MRI voxel index (discretized): i ∈ {1, 2, …, N},  N = Nx×Ny×Nz

# 2. State Variables

The system state is a pair of spatially-distributed fields:

## 2.1 Temperature Field

T=Tr,t:Ω×0,tf→R°C or K

T(r, t) is the primary observable — it is measured in real time by MRI thermometry. In the discretized system, the temperature state vector is:

xTt=T1t,T2t,…,TNtᵀ∈Rᴺ

where Tᵢ(t) = T(rᵢ, t) is the temperature at the i-th voxel centroid rᵢ

## 2.2 Thermal Damage Field (Arrhenius)

Ωd=Ωdr,t:Ω×0,tf→R+dimensionless

Ωd accumulates irreversible tissue damage. It is defined by the Arrhenius thermal dose model:

Ωdr,t=0tA·exp-Ea/R·Tr,τdτ

Equivalently in ODE form:

dΩddt=A·e-Ea/R·T ,Ωdr,0=0

where

A [s⁻¹]: frequency factor

Ea [J/mol]: activation energy

R=8.314 [ J/(mol·K)]: universal gas constant

The threshold Ωd=1 corresponds to 63% probability of cell death (irreversible necrosis).

## 2.3 Combined Distributed State

The complete infinite-dimensional state is:

Xr,t=Tr,t,Ωdr,t,r∈Ω,t∈0,tf

After spatial discretization, the lumped finite-dimensional state vector is:

xt=xTtᵀ,xΩtᵀᵀ∈R2ᴺ

# 3. Control Variables

The control input depends on the ablation modality. We consider the general microwave/RF case:

## 3.1 Scalar Power Control (Single Applicator)

ut=Pt∈0,PmaxW

## 3.2 Vector Control (Robotic MRI-Guided Ablation)

For a robotic system with actuated probe position and orientation:

ut=Pt,qtᵀ∈R1+nq

where qt∈Rnq is the robot joint configuration vector.

## 3.3 Control Constraints

ut∈U≔u:0≤Pt≤Pmax,ut≤u̇max

The control set 𝒰 encodes physical actuator limits (maximum power, maximum slew rate, probe mechanical limits).

# 4. System Dynamics (State Equations)

## 4.1 Pennes Bioheat Equation (Continuous PDE)

The temperature field is governed by the Pennes bioheat equation:

ρrcr∂T∂t=∇·kr∇T-ωbrρbcbT-Tb+Qmetr+Qsourcer,u,t

where the terms are defined in the following table:

- ρr [kg/m³]: tissue mass density

- cr [J/(kg·K)]: tissue specific heat capacity

- kr [W/(m·K)]: thermal conductivity (possibly anisotropic tensor)

- ωbr [s⁻¹]: blood perfusion rate

- ρb = 1050 [kg/m³]: blood density

- cb = 3600 [J/(kg·K)]: blood specific heat

- Tb = 37 [°C]: arterial blood temperature (body core, ~37°C)

- Qmetr [W/m³]: metabolic heat generation rate

- Qsourcer,u,t [W/m³]: volumetric heat deposition from ablation source

## 4.2 Microwave Heat Source Model

For microwave ablation, the heat source is proportional to the Specific Absorption Rate (SAR):

Qsourcer,u,t=SARr,qt·Pt=σrEr,qt2·Pt

where σr [S/m] is tissue electrical conductivity and Er,qt [V/m] is the normalized electric field pattern (precomputed from a finite element EM model). The spatial pattern b(r, q(t)) encodes the antenna near-field distribution:

Qsourcer,u,t=b(r, q) · P(t)

## 4.3 Damage Dynamics

∂Ωd∂t=A·e-Ea/R·Tr,t,r∈Ω,t∈0,tf

## 4.4 Discretized State-Space Form

Applying finite difference or finite element spatial discretization (mesh size h, N voxels) yields the ODE system:

ẋt=fxt,ut=AdxTt+But+dxTt

where:

- Ad ∈ ℝᴺˣᴺ: sparse diffusion-perfusion matrix 
(tri- or penta-diagonal for FD, or FEM stiffness/mass matrix)

- But ∈ ℝᴺ: control input vector, Bi=brᵢ,q·P

- dxTt∈ ℝᴺ: Arrhenius damage rate vector, dᵢ=A·e-Ea/R·Tr,t

Explicitly:

ẋT=1ρc∙KdxT-WbxT-Tb1+Qmet+BPPt

ẋΩ=A·e-Ea/R·xT

[elementwise]

where Kd ∈ ℝᴺˣᴺ is the (FD or FEM) discrete Laplacian weighted by kr, and

Wb= diag(ω_b,i ρ_b c_b)

# 5. Boundary and Initial Conditions

## 5.1 Initial Conditions

Tr,0=T0r,r∈Ωpre-treatmentbaselinetemperature

Ωdr,0=0,r∈Ωnopre-existingthermaldamage

Typically T₀(r) ≈ 37°C (body temperature) throughout the domain, or can be the measured pre-treatment MRI thermometry map.

## 5.2 Boundary Conditions on ∂Ω

Three standard options (select based on anatomy):

### Dirichlet (Fixed Temperature — large vessel or body surface):

Tr,t=T∞,r∈∂ΩD

### Neumann (Insulated — symmetry plane):

kr∂T∂n=0,r∈∂ΩN

### Robin (Convective Cooling — tissue-air or tissue-fluid interface):

kr∂T∂n+hcT-T∞=0,r∈∂ΩR

where

hc [W/(m²·K)]: convective heat transfer coefficient

T∞ [°C or K]: ambient temperature

6. Domain Partition

The tissue domain Ω is partitioned into three disjoint non-overlapping regions:

Ω=ΩT∪ΩH∪ΩM,ΩT∩ΩH=ΩT∩ΩM=ΩH∩ΩM=∅

- ΩT ⊂ Ω: tumor (target) region — to be fully ablated

- ΩH ⊂ Ω: healthy tissue — to be protected

- ΩM ⊂ Ω: safety margin / transition zone (optional)

These regions are defined from pre-treatment MRI segmentation.

# 7. Optimal Control Problem Formulation

## 7.1 Standard Bolza Form

The complete optimal control problem is stated in Bolza form:

minu·,tfJu,tf=Φxtf,tf+0tfLxt,ut,tdt

subject to the state dynamics, boundary conditions, control constraints, and terminal constraints detailed below.

## 7.2 Running Cost L (Lagrangian)

The running cost captures three competing objectives during the treatment:

Lx,u,t=α1ut2+α2ΩHmax0,Tr,t-Tsafe2dr+α3

- α1ut2: penalize excessive energy deposition 
(reduce tissue damage, device wear)

- α2ΩHmax0,Tr,t-Tsafe2dr: quadratic penalty for healthy tissue temperature exceeding threshold Tsafe≈ 42 - 45 °C

- α3>0: constant rate cost on time; encourages minimum-time treatment

## 7.3 Terminal Cost Φ (Mayer Term)

The Mayer term penalizes incomplete tumor ablation at the final time tf:

Φxtf,tf=γ1ΩTmax0,1-Ωdr,tf2dr+γ2tf

- γ1ΩTmax0,1-Ωdr,tf2dr: soft penalty for unablated tumor voxels

- γ2tf: direct penalization of total procedure time

## 7.4 Complete Cost Functional

Combining the Lagrangian and Mayer terms:

Ju,tf=γ1ΩTmax0,1-Ωdr,tf2dr+γ2tf+0tfα1ut2+α2ΩHmax0,Tr,t-Tsafe2dr+α3dt

The weights α₁, α₂, α₃, γ₁, γ₂ > 0 are tuning parameters reflecting clinical priorities.

# 8. Constraints

## 8.1 State Dynamics (Equality Constraint)

ẋt=fxt,ut,t∈0,tf, x0=x0=T0ᵀ,0ᵀᵀ

## 8.2 Path (State-Control) Inequality Constraints

These must be held for all times t∈0,tf and all spatial locations:

### (a) Healthy tissue temperature limit:

Tr,t≤Tsafe,∀r∈ΩH,∀t∈0,tf

### (b) Absolute safety temperature (critical structures):

Tr,t≤Tcrit,∀r∈Ωcrit⊂ΩH,∀t∈0,tf

### (c) Control box constraints:

0≤Pt≤Pmax,∀t∈0,tf

### (d) Control rate constraints (actuator slew limit):

dPdt≤Ṗmax,∀t∈0,tf

## 8.3 Terminal (Endpoint) Constraints

### (a) Tumor ablation requirement (hard constraint or soft via Φ):

Ωdr,tf≥1,∀r∈ΩT

### (b) Free final time positivity:

tf>0

### (c) Maximum allowable treatment time:

tf≤Tmax

## 8.4 Summary: Complete Constrained Problem

minu· ∈ U,  tf ∈ 0,TmaxJ[u,tf] subject to:

ẋ=fx,u,x0=x0                                   [dynamics]

Tr,t≤Tsafe,∀r∈ΩH,t∈0,tf         [path safety]

0≤Pt≤Pmax                                    [control bounds]

Ωdr,tf≥1,∀r∈ΩT                 [terminal ablation]

# 9. Parameter Summary

Table 1 below summarizes key physical parameters and their typical values for soft tissue ablation:

- ρ: tissue density — 1000–1100 kg/m³

- c: specific heat — 3400–3800 J/(kg·K)

- k: thermal conductivity — 0.4–0.6 W/(m·K)

- ω_b: blood perfusion rate — 0.003–0.01 s⁻¹

- T_b: blood temperature — 37°C

- A (Arrhenius): frequency factor — 3.1×10⁹⁸ s⁻¹ (for protein denaturation)

- E_a (Arrhenius): activation energy — 6.28×10⁵ J/mol

- R: universal gas constant — 8.314 J/(mol·K)

- T_safe: healthy tissue safety threshold — 42–45°C

- T_crit: critical structure absolute limit — 40°C

- P_max: maximum applicator power — 30–100 W

- T_max: maximum allowed procedure time — 10–30 min

# References

- Pennes, H.H. (1948). Analysis of tissue and arterial blood temperatures in the resting human forearm. J. Appl. Physiol., 1(2), 93–122.

- Moritz, A.R., Henriques, F.C. (1947). Studies of thermal injury II. Am. J. Pathol., 23(5), 695–720. [Arrhenius damage model]

- Bett, S., et al. (2022). Model predictive control for MRI-guided thermal ablation. IEEE Trans. Biomed. Eng.

- Liberzon, D. (2012). Calculus of Variations and Optimal Control Theory. Princeton University Press.

- Biegler, L.T. (2010). Nonlinear Programming: Concepts, Algorithms, and Applications to Chemical Processes. SIAM.

- Kirk, D.E. (1970). Optimal Control Theory: An Introduction. Dover.